# Cheat Sheet – Road Transport

Table 11: Parameters used to estimate the total kilometer travelled with technology categorization

| Vehicle | Fuel | Fraction for type of gas | Km/day | Km/year | Km/Litre | Litre/km |
| --- | --- | --- | --- | --- | --- | --- |
| Motor cycle | Petrol | 1 | 13 | 4745 | 15 | 0.0667 |
| Car « | Petrol | 1 | 22 | 8030 | 12 | 0.11 |
| Taxis « | Petrol | 1 | 100 | 36500 |  | 0.11 |
| Bus v | Petrol | 0.023 | 40 | 14600 | 5 | 0.164 |
|  | Diesel | 0.977 | 40 | 14600 | 8 | 0.42 |
| Pick-up ∆ | Petrol | 0.3208 | 20 | 7300 | 7 | 0.143 |
|  | Diesel | 0.678 | 20 | 7300 | 10 | 0.125 |
| Lorry v | Petrol | 0.021 | 40 | 14600 | 5 | 0.22 |
|  | Diesel | 0.979 | 40 | 14600 | 8 | 0.3 |
| Van ∆ | Petrol | 0.4819 | 40 | 14600 | 5 | 0.142 |
|  | Diesel | 0.5178 | 40 | 14600 | 8 | 0.125 |
| Jeep ∆ | Petrol | 0.7337 | 20 | 7300 | 7 | 0.142 |
|  | Diesel | 0.2663 | 20 | 7300 | 10 | 0.125 |
| Truck v | Petrol | 0.0383 | 40 | 14600 | 5 | 0.22 |
|  | Diesel | 0.9616 | 40 | 14600 | 8 | 0.3 |
| Other vehicles v | Diesel | 1 | 5 | 1825 | 8 | 0.3 |

Table 9: Emission factors for road transport

|  | Emission Factor (EF) kg/TJ |  |  |
| --- | --- | --- | --- |
| Fuel | CO2 | CH4 | N2O |  |
| Gas/Diesel Oil | 74100 | 3.9 | 3.9 |  |
| Petrol | 69300 | 33 | 3.2 |  |

- To calculate the total fuel use using VKT, the following formula can be used.

Total fuel use (L)= N.V∗R.T.F∗K.D∗365∗L.KTotal fuel use L= N.V∗R.T.F∗K.D∗365∗L.K

Where,

N.V = Number of vehicles (by type of vehicle)

R.T.F = Ratio by type of fuel of the vehicle (ratio of vehicles using petrol or diesel by the same category of vehicles. e.g., Pick-up using petrol and diesel)

K.D = Kilometers per day (parameter from Table 11)

L.K = Litres per kilometer

- Once the total fuel consumption is obtained in litres, it needs to be converted to mass (Gg) using the respective density conversion using

Diesel density = 0.84 (kg/L) or (T/m3)

Petrol density = 0.75 (kg/L) or (T/m3)

Total fuel use (Gg)= Total fuelLitres∗Densityfuel /1000,000Total fuel use Gg= Total fuelLitres∗Densityfuel /1000,000

- The total fuel use (Gg) with the NCV and emission factors in Table 9 could be used to calculate the emissions for the respective fuel type.

Emissions in (Gg)= Total fuel massGg∗NCVof the fuel ∗EFof the fuel /1000,000Emissions in Gg= Total fuel massGg∗NCVof the fuel ∗EFof the fuel /1000,000

- Once the emissions in mass value are obtained for the respective gases, the CO2 and non-CO2 gases need to be converted to CO2 eq to get the final emission in CO2 equivalent.